# CS Acrovyn ® Model P-OWM/P-RWA

Suggested Specifications | Section 10 26 00

# CS Acrovyn® Models P-OWM, P-RWM and P-RWMSS

Part 1 - General

- Summary

  - This section includes the following types of wall protection systems:

    - Handrails

  - Related sections: The following sections contain requirements related to this section:

    - Corner Guards, Bumper Guards, Crash Rails, Accent Rails, Wall Covering, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

    - Blocking in walls for fasteners; refer to section 09 22 00 “Supports for Plaster and Gypsum Board”

- References

  - National codes (IBC, UBC, SBCCI, BOCA, OSHA, Life Safety, OSHPD and ADA)

  - American Society for Testing and Materials (ASTM)

- Submittals

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of handrails. Show methods of attachment to adjoining construction.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, finish and end cap attachment and alignment:

    - 12" (304.8mm) long sample of each model specified including end cap.

  - Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- Quality Assurance

  - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer’s qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA, OSHA, Life Safety, OSHPD and ADA.

  - Fire performance characteristics: Provide wood and metal components tested in accordance with ASTM E84 for Class A/1 fire characteristics.

  - Impact Strength: Provide assembled wall protection units that have been tested in accordance with the applicable provisions of ASTM F476.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- Delivery, Storage and Handling

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Store materials in original, undamaged packaging in a cool, dry place out of direct sunlight and exposure to the elements. A minimum room temperature of 40°F (4°C) and a maximum of 100°F (38°C) should be maintained.

  - Material must be stored flat.

- Project Conditions

  - Materials must be acclimated in an environment of 65°-75°F (18°-24°C) for at least 24 hours prior to beginning the installation.

  - Installation areas must be enclosed and weatherproofed before installation commences.

Part 2 - Products

- Manufacturers

  - Interior surface protection products specified herein and installed on the submittal drawings shall be manufactured by Construction Specialties, Inc.

- Materials

  - Solid Wood Components: Shall be manufactured from plain sawn, FAS grade hardwood, kiln dried to a moisture content of 6% to 10%.

  - Aluminum: Extruded aluminum should be 6063-T6 alloy Crash rail to be nominal .080" (2.03mm) thickness. Minimum strength and durability properties as specified in ASTM B221.

  - Stainless Steel: Cast brackets to be type 304 alloy with #4 satin finish.

  - Fasteners: All fasteners to be non-corrosive and compatible with aluminum retainers.  All necessary fasteners to be supplied by the manufacturer.

- Handrails

  - Handrails to be CS Acrovyn: Surface mounted handrail and crash rail configuration with end caps returning to the wall. Attachment hardware shall be appropriate for wall construction.

    - Model P-OWM  6" (152.4mm) high configuration consisting of an oval wood handrail, an aluminum crash rail and matching end caps returning to the wall. Select handrail from one of Renaissance™ wood species and finishes. FSC certified wood can be specified. Crash rail to be powder coated aluminum; select from standard powder coat finishes. Dual cantilevered mounting brackets to be stainless steel available with optional powder coat; select from standard powder coat finishes. (Specifier note: This product is Cradle to Cradle Certified™ Silver. Cradle to Cradle Certified™ is a certification mark licensed by the Cradle to Cradle Products Innovation Institute.)

    - Model P-RWM  6 3/8" (161.9mm) high configuration consisting of a round wood handrail, an aluminum crash rail and matching end caps returning to the wall. Select handrail from one of Renaissance™ wood species and finishes. FSC certified wood can be specified. Crash rail to be powder coated aluminum; select from standard powder coat finishes. Dual cantilevered mounting brackets to be stainless steel. For optional stainless steel end caps specify P-RWMSS. Optional 90 degree stainless steel outside corners and 3" (76.2mm) splices available. All stainless steel components available with optional powder coat; select from standard powder coat finishes. [Specifier note: This product is Cradle to Cradle Certified™ Silver (exceptions: stainless steel end caps, corners and splices). Cradle to Cradle Certified™ is a certification mark licensed by the Cradle to Cradle Products Innovation Institute.]

- Finishes

  - All wood components shall be factory finished. Wood components to be final coated with water based, high solids, clear lacquer using a two coat process. Finish shall be in accordance with specified AWI finish system. Coverage shall be a minimum of 3-5 mils. Gloss shall be measured on 60° gloss meter as per ASTM D523.

  - General: Comply with NAAMM “Metal Finishes Manual” for recommendations relative to applications and designations of finishes.

- Fabrication

  - General: Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish and member sizes.

  - Preassemble wood components in shop as much as possible to minimize field assembly.

  - Fabricate components with wood joints lightly chamfered. Provide surfaces free of chipping, dents and other imperfections.

Part 3 - Execution

- Examination

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- Preparation

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

- Installation

  - Install the work of this section in strict accordance with the manufacturer's recommendations using only approved mounting hardware and locating all components firmly into position, level and plumb.

  - Temperature at the time of installation must be between 65°-75°F (18°-24°C) and be maintained for at least 48 hours after the installation.

- Cleaning

  - General: Immediately upon completion of installation, clean rails and accessories in accordance with manufacturer’s recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- Protection

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.