# | Suggested Specifications | Section 10 26 00

CS Acrovyn® Model ECR-20S

Part 1 - General

  - Summary

    - This section includes the following types of wall protection systems:

      - Crash Rails

    - Related sections: The following sections contain requirements related to this section:

      - Handrails, Corner Guards, Bumper Guards, Accent Rails, Wall Covering, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

      - Blocking in walls for fasteners; refer to section 09 22 00 “Supports for Plaster and Gypsum Board”

# References

    - National codes (IBC, UBC, SBCCI, BOCA and Life Safety)

    - American Society for Testing and Materials (ASTM)

# Submittals

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

    - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

    - Shop drawings showing locations, extent and installation details of crash rails. Show methods of attachment to adjoining construction.

    - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification:

      - 12" (304.8mm) long sample of each model specified.

    - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

# Quality Assurance

    - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

    - Manufacturer’s qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

    - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA and Life Safety.

    - Fire performance characteristics: Provide metal components tested in accordance with ASTM E84 for Class A/1 fire characteristics.

    - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

# Delivery, Storage and Handling

    - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

    - Material must be stored flat.

# Project Conditions

    - Installation areas must be enclosed and weatherproofed before installation commences.

# Part 2 - Products

  - Manufacturers

    - Interior surface protection products specified herein and included on the submittal drawings shall be

manufactured by Construction Specialties, Inc.

# Materials

    - Stainless steel: To be type 304 alloy with #4 satin finish, 1/4" (6.35mm) thickness; minimum strength and durability properties as specified in ASTM A276.

    - All necessary fasteners to be supplied by the manufacturer.

# Crash Rails

    - Heavy duty crash rails to be by CS Acrovyn: Extended mount crash rail lengths to be supplied prefabricated with corners and end returns formed. End returns and corners shall not be separate pieces. All units pre-drilled. Mounting hardware shall be supplied by the manufacturer.

# Model ECR-20S Stainless steel crash rail 2" (50.8mm) high x 1/4" (6.4mm) thick with continuous 2" (50.8mm) radiused ends standard, 135° corners available. Outside rail surface shall be no more than 3 1/4" (82.6mm) from wall mounting surface. Stainless pipe bracket standard. Optional powder coat finish available; select from standard powder coat finishes.

  - Finishes

    - General: Comply with NAAMM "Metal Finishes Manual" for recommendations relative to applications and designations of finishes.

# Fabrication

    - General: Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish and sizes. All based upon required field verified dimensions.

# Part 3 - Execution

  - Examination

    - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

      - Do not proceed until unsatisfactory conditions have been corrected.

# Preparation

    - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

    - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

# Installation

    - Install the work of this section in strict accordance with the manufacturer's recommendations and the required field verified dimensions.

    - Use only approved mounting hardware, and locating all components firmly into position, level and plumb.

# Cleaning

    - General: Immediately upon completion of installation, clean material in accordance with manufacturer’s recommended cleaning method.

    - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

# Protection

    - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.