# Surface Mounted Cubicle Track and Carriers

Suggested Specifications | Section 10 21 24

Part 1 – General

  - Summary

- Section includes:

  - Ceiling mounted curtain track

  - Track accessories and attachments

  - Submittals

- Product Data: Manufacturer’s product data sheets for all specified curtain track.

- Detail Drawings: Mounting details with the appropriate fasteners for specified project substrates.

- Samples: Verification samples of cubicle track, 4" (102mm) in length.  Complete with (1) carrier as

specified and stop.

- Manufacturer’s standard installation instructions

  - Delivery, Storage, and Handling

- Deliver materials in unopened factory packaging.

- Inspect material on delivery to verify products are as specified.

  - Warranty

- Manufacturer’s standard warranty against manufacturer defects.

Part 2 - Products

  - Acceptable Manufacturers

- Subject to compliance with the requirements listed, provide cubicle track and carriers from the following manufacturer:

- CS Cubicle Curtains

3 Werner Way

Lebanon, NJ 08833

P: (800) 233-8493

CCInfo@c-sgroup.com

www.c-sgroup.com

- Substitutions: Alternative manufacturers will not be accepted

  - Cubicle Track System

- Cubicle Tracks: CS Cubicle Curtains #6063 suspended-mounted tracks of heavy extruded aluminum alloy 6063-T5, 1 3/8" x 3/4", slotted to receive roller carriers, complete with accessories and components required for complete and secure installations including splices, end caps and corner bends.

  - Suspension Height:  ¾” square aluminum hangers with suspension brackets made of molded nylon to adapt to CS 6063 aluminum track.

      - Suspension bar height to be: _____ (insert height of suspension here)

  - Corner Bends:  Corner bends up to 36” radius are to be fabricated in one continuous “L” shape.  Radiuses above 36” to be continuous or spliced based on room condition.

    - Finish:  
       <**Note to specifier – choose desired track finish>

      - Clear anodized aluminum [OR]

      - White powder coat finish

- Carriers:

<**Note to specifier - choose desired carrier for project (1, 2, 3, 4, or combination of)>

  - CS Cubicle Curtains 1062N, virgin nylon axle with nylon wheels complete with nickel-plated brass bead-chain and hook assembly.

    - Provide one carrier for each 6" of cubicle curtain width.

  - CS Cubicle Curtains Breakaway Carrier SBC30 designed to pop out on one side when 8 lbs of pressure is applied. Beaded nylon carrier.

    - Provide one carrier for each 6" of cubicle curtain width.

  - CS Cubicle Curtains SP062, spool carrier with all nylon spool, nickel plated brass bead connector, and aluminum hook.

    - Provide one carrier for each 6” of cubicle curtain width.

  - Breakaway Carriers:  CS Cubicle Curtains 1075P, all nylon breakaway carriers. Carrier to have virgin nylon hook, axle and nylon wheels. Carrier hook is to be designed to break away from axle at 8 to 10 pounds of applied pressure.

    - Provide one carrier for each 6" of cubicle curtain width

END OF SECTION