CS Acrovyn ® Model P-OM

Suggested Specifications | Section 10 26 00

Canadian Specifications

Part 1 - General

- Summary

  - This section includes the following types of wall protection systems:

    - Handrails

  - Related sections: The following sections contain requirements related to this section:

    - Bumper Guards, Crash Rails, Corner Guards, Accent Rails, Wall Covering, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

    - Blocking in walls for fasteners; refer to section 09 22 00 “Supports for Plaster and Gypsum Board”

- References

  - National Building Code of Canada (NBC 2010)

  - National codes (IBC, UBC, SBCCI, BOCA, OSHA, Life Safety, OSHPD and ADA)

  - American Society for Testing and Materials (ASTM)

- Submittals

  - General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”.

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of handrails. Show methods of attachment to adjoining construction.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, finish and end cap attachment and alignment.

    - 9" (228.6mm) long sample of each model specified including end cap and mounting hardware.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- Quality Assurance

  - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer’s qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies should conform to all applicable codes including NBC 2010, IBC, UBC, SBCCI, BOCA, OSHA, OSHPD, ADA and CA 01350.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- Delivery, Storage and Handling

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Store materials in original, undamaged packaging in a cool, dry place out of direct sunlight and exposure to the elements. A minimum room temperature of 40°F (4°C) and a maximum of 100°F (38°C) should be maintained.

  - Material must be stored flat.

- Project Conditions

  - Materials must be acclimated in an environment of 65°-75°F (18°-24°C) for at least 24 hours prior to beginning the installation.

  - Installation areas must be enclosed and weatherproofed before installation commences.

- Warranty

  - Acrovyn 5-year Limited Warranty

    - Applies to Interior Wall Protection orders that do not include recommended components accessories

      - Assemblies = Brackets, Hardware

      - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

  - Limited Lifetimes System Warranty

    - Applies to CS Interior Wall Protection projects that include all recommended components and accessories related to CS Interior Wall Protection Products.

      - Assemblies = Brackets, Hardware

      - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

# Part 2 - Products

- Manufacturers

  - Interior surface protection products specified herein and included on the submittal drawings shall be manufactured by Construction Specialties, Inc., 3 Werner Way, Lebanon, NJ 08833 USA 800-233-8493; email: cet@c-sgroup.com

  - Drawings and specifications are based on manufacturer’s literature from Construction Specialties, Inc. drawings and specifications unless otherwise indicated. Other manufacturers must be approved equal by Architect/Owner.

- Materials

  - Aluminum: Extruded aluminum components should be 6063-T6 alloy, nominal .060" (1.52mm) thick aluminum handrail. Minimum strength and durability properties as specified in ASTM B221. Cast brackets to be Powder Coated Aluminum.

  - Fasteners: All fasteners to be non-corrosive and compatible with aluminum retainers.  All necessary fasteners to be supplied by the manufacturer.

- Handrails

  - Handrails to be Acrovyn by Construction Specialties: Surface mounted handrail with end caps returning to the wall and mounting brackets designed to meet Canadian code required 50mm minimum offset spaced as indicated on C/S installation instructions. Attachment hardware shall be appropriate for wall construction.

    - Model P-OM  5 1/2" h (139.4mm) handrail configuration consisting of oval aluminum handrail with matching end caps returning to the wall. Aluminum handrail and Single cantilevered mounting brackets to be powder coated; select from one of (8) powder coat finishes.

- Finishes

  - General: Comply with NAAMM “Metal Finishes Manual” for recommendations relative to applications and designations of finishes.

- Fabrication

  - General: Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish and member sizes.

  - Preassemble wood components in shop as much as possible to minimize field assembly.

  - Fabricate components with wood joints lightly chamfered. Provide surfaces free of chipping, dents and other imperfections.

Part 3 - Execution

- Examination

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- Preparation

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

- Installation

  - Install the work of this section in strict accordance with the manufacturer's recommendations, using only approved mounting hardware, and locating all components firmly into position, level and plumb.

  - Temperature at the time of installation must be between 65°-75°F (18°-24°C) and be maintained for at least 48 hours after the installation.

- Cleaning

  - General: Immediately upon completion of installation, clean rails and accessories in accordance with manufacturer’s recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- Protection

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.