# Surface Mounted I.V. Track and Pendants

Suggested Specifications | Section 10 21 23

Part 2 – Products

  - Acceptable Manufacturers

- Subject to compliance with the requirements listed, provide cubicle curtains from the following manufacturer:

- CS Cubicle Curtains, a Division of Construction Specialties, Inc.

3 Werner Way

Lebanon, NJ 08833

P:  (908) 849-4000 / F:  (908) 849-4398

CCInfo@c-sgroup.com

www.c-sgroup.com

- Alternative manufacturers will not be accepted.

  - IV Track System

- I.V. Tracks:  CS Cubicle Curtains I.V. track model #17000. Surface-mounted tracks of heavy extruded aluminum alloy 6063-T5, 1 ⅜" x ¾", slotted to receive twist lock I.V. Carriage, compete with accessories and components required for complete and secure installations including splicers, end caps and corner bends.

  - Corner Bends:  Shall have a 12” - 18" radius fabricated in one continuous “L” shape.

  - Finish:  Clear anodized aluminum.

- Carriers:  C/S Cubicle Curtains 17500 Twist-Lock I.V. Carriage, Carrier mechanism shall be designed to lock against surface of reinforcing I.V. Track.

- Provide one carrier per unit of I.V. Track, or as needed.

- I.V. Pendants:

- Model # 17100 – Standard 5 Bottle I.V. Pendant, Fabricated of polished stainless steel 9/16" diameter tubing for the main shaft and four ¼" stainless steel folding arms. This I.V. Unit will be able to be raised or lowered to a height from 19" to 32" to adjust to most ceiling heights. The locking mechanism is a stainless steel thumb push at base of unit.

- Model # 17800 – Deluxe 9 Bottle I.V. Pendant, Fabricated of polished stainless steel this unit has a ¾" diameter tubing for the main shaft, 3/8" diameter for the inner shaft, and eight folding ¼" diameter spiral safety arms. This I.V. Unit will be able to be raised or lowered to a height from 28" to 42" to adjust to most ceiling heights.

END OF SECTION