# Stainless Steel Sheet

# Wall Covering

Suggested Specifications | Section 10 26 00

PART 1 – GENERAL

- SUMMARY

  - This section includes the following types of wall protection systems:

    - Wall Covering

  - Related sections: The following sections contain requirements related to this section:

    - Handrails, Corner Guards, Bumper Guards, Crash Rails, Accent Rails, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

- REFERENCES

  - National codes (IBC, UBC, SBCCI, BOCA and Life Safety)

  - American Society for Testing and Materials (ASTM)

- SUBMITTALS

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of wall covering products.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, texture, pattern and thickness:

    - Sample of each product specified.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- QUALITY ASSURANCE

  - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer's qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA and Life Safety.

  - Fire performance characteristics: Provide stainless steel components tested in accordance with ASTM E84 for Class A/1 fire characteristics.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- DELIVERY, STORAGE AND HANDLING

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Materials must be stored flat.

- PROJECT CONDITIONS

  - Installation areas must be enclosed and weatherproofed before installation commences.

PART 2 – PRODUCTS

- MANUFACTURERS

  - Interior surface protection products specified herein and included on the submittal drawings shall be manufactured by Construction Specialties, Inc.

- MATERIALS

  - Stainless Steel: Material to be 16-gauge type 304 alloy with #4 satin finish.

- WALL COVERING

  - Stainless Steel Sheet to be CS: Nominal .0625" (1.59mm) thick stainless steel sheet with standard Smooth texture. Supplied in 4' x 8' or 10' (1.2m x 2.4m or 3.0m) sheet sizes. Diamond Plate texture available.

- FABRICATION

  - General: Fabricate wall protection products to comply with requirements indicated for design, dimensions, detail, finish and sizes.

- FINISHES

  - General: Comply with NAAMM "Metal Finishes Manual" for recommendations relative to applications and designations of finishes.

- ACCESSORIES

  - Mounting: Stainless steel wall protection shall be furnished as a complete packaged system, including appropriate adhesive or mechanical fasteners.

PART 3 – EXECUTION

- EXAMINATION

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- PREPARATION

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer's installation instructions.

- INSTALLATION

  - Install the work of this section in strict accordance with the manufacturer's recommendations using only approved hardware and locating all components firmly into position, level and plumb.

- CLEANING

  - General: Immediately upon completion of installation, clean wall protection products and accessories in accordance with manufacturer's recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- PROTECTION

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.

END OF SECTION