# Acrovyn by Design® Tapestry Collection

# Wall Covering Sheet

CS Acrovyn® Sheet with up to 50% Post-Consumer Recycled Content

Suggested Specifications | Section 10 26 00

PART 1 – GENERAL

- SUMMARY

  - This section includes the following types of wall protection systems:

    - Wall Coverings

  - Related sections: The following sections contain requirements related to this section:

    - Handrails, Corner Guards, Bumper Guards, Crash Rails, Accent Rails, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

- REFERENCES

  - American Society for Testing and Materials (ASTM)

- SUBMITTALS

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of wall covering products.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, texture, pattern and thickness:

    - Sample of each product specified available in 4” x 4” standard size.

  - Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- QUALITY ASSURANCE

  - Installer qualifications: Engage an installer who has no less than 3 years’ experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer qualifications: Not less than 5 years’ experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA, Life Safety.

    - Flame spread: 26 – 75

    - Smoke developed: 450 or less

  - Fire performance characteristics: Provide engineered PVC Free wall protection system  indicating that they are identical to those tested in accordance with ASTM E84 for B/2 characteristics listed below: 
Thatched pattern.

    - Flame spread: 26 – 75

    - Smoke developed: 450 or less

  - Chemical and stain resistance: Provide wall protection system components with chemical and stain resistance in accordance with ASTM D543.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- DELIVERY, STORAGE AND HANDLING

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Store materials in undamaged packaging in a clean, dry place out of direct sunlight and exposure to the elements. A minimum room temperature of 40ºF (4ºC) and a maximum of 100ºF (38ºC) should be maintained.

  - Materials must be stored flat.

- PROJECT CONDITIONS

  - Materials must be acclimated in an environment of 65-76ºF (18-24ºC) for at least 24 hours prior to beginning the installation.

  - Installation areas must be enclosed and weatherproofed before installation commences.

- WARRANTY

  - Acrovyn 5-year Limited Warranty

- Applies to Interior Wall Protection orders that do not include recommended components or accessories

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

  - Limited Lifetime Systems Warranty

- Applies to CS Interior Wall Protection projects that include all recommended components and accessories related to CS Interior Wall Protection Products.

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

PART 2 – PRODUCTS

- MANUFACTURERS

  - Interior surface protection products specified herein and included on the submittal drawings shall be manufactured by Construction Specialties, Inc., 3 Werner Way, Lebanon, NJ 08833 USA 800-233-8493; email: cet@c-sgroup.com

  - Drawings and specifications are based on manufacturer’s literature from Construction Specialties, Inc. drawings and specifications unless otherwise indicated. Other manufacturers must be approved equal by Architect/Owner.

- MATERIALS

  - Engineered PVC Free with up to 50% Post-Consumer Recycled Content: Rigid sheet should be high-impact Acrovyn by Design with standard Suede texture, nominal .040" (1.02mm) thickness.  Tapestry is a fused sheet product with an opaque backer.

  - Aluminum: Optional aluminum trims to be alloy 6063 T5 with clear or colored anodized finish; minimum strength and durability properties as specified in ASTM B221.  The color anodized finish is available in eight colors and is not covered under 1.04.G.

  - Cured ColorFlex® II Bacteria Resistant Caulk has a Shore A value of greater than 55 (product considered “pick-resistant” based on industry standards) testing conducted by independent lab in accordance with ASTM C661 and ASTM C920.

- WALL COVERING

  - Engineered PVC Free rigid sheet to be Acrovyn by Design Tapestry Collection (with up to 50% post-consumer recycled content):

**Note to Specifier – update appropriate section in red below and delete**

    - Sheet Size:

      - 4’ x 8’ (1.2m x 2.4m) sheet size

    - Finishes:

      - Select from one of the nine color options in Thatched pattern.

        - (Thatched) (color name)

        - Other colors will not be accepted as “closest standard color” must match exactly.

      - or CS Acrovyn by Design Tapestry Thatched and Color to be indicated in the finish schedule from one of manufacturer’s available colors.

    - Specify color-matched caulk, clear caulk, Acrovyn trims or aluminum trims as needed for joints/transitions. [Specifier note: refer to the Acrovyn Cradle to Cradle Product Summary at www.acrovyn.com/c2c to determine which colors and patterns are Cradle to Cradle Certified™ Gold or Silver (exceptions: adhesive, primer and caulk). Cradle to Cradle Certified™ is a certification mark licensed by the Cradle to Cradle Products Innovation Institute.]

      - Aluminum trims are recommended. See aluminum trim finish options.

- FABRICATION

  - General: Fabricate wall covering to comply with requirements indicated for design, dimensions, detail, finish and sizes.

- FINISHES

  - General: Comply with NAAMM "Metal Finishes Manual" for recommendations relative to applications and designations of finishes.

- ACCESSORIES

  - Adhesive: Acrovyn wall covering shall be furnished as a complete packaged system, including appropriate standard adhesive.

  - Primer, caulk and trims available for purchase.

PART 3 – EXECUTION

- EXAMINATION

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- PREPARATION

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions. Minimum Level 3 wall finish is required; for surfaces with Level 5 finish, ensure the surface and any surface coatings are fully dry and cured.

  - Preparation: Take all necessary steps to prevent damage to material during installation as required in manufacturer's installation instructions.

- INSTALLATION

  - Install the work of this section in strict accordance with the manufacturer's recommendations using approved adhesive. Note there are special installation instructions for non-standard conditions: radius walls, tile, CMU block, etc.

  - Temperature at the time of installation must be between 65-75ºF (18-24ºC) and be maintained for at least 48 hours after the installation to allow for proper adhesive set-up.

  - Relative humidity shall not exceed 80%.

  - Do not expose wall covering to direct sunlight during or after installation. This will cause the surface temperature to rise, which in turn will cause bubbles and delamination.

- CLEANING

  - General: Immediately upon completion of installation, clean material in accordance with manufacturer's recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- PROTECTION

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.

END OF SECTION