# | Suggested Specifications | Section 10 26 00

# CS Acrovyn® Model P-OW, P-RW and P-RWSS

Part 1 - General

- Summary

  - This section includes the following types of wall protection systems:

    - Handrails

  - Related sections: The following sections contain requirements related to this section:

    - Corner Guards, Bumper Guards, Crash Rails, Corner Guards, Accent Rails, Wall Covering, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

    - Blocking in walls for fasteners; refer to section 09 22 00 “Supports for Plaster and Gypsum Board”

- References

  - National codes (IBC, UBC, SBCCI, BOCA, OSHA, Life Safety, OSHPD and ADA)

  - American Society for Testing and Materials (ASTM)

  - California 01350 Specification

- Submittals

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent, and installation details of handrails. Show methods of attachment to adjoining construction.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, finish and end cap attachment and alignment:

    - 12" (304.8mm) long sample of each model specified including end cap.

  - Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- Quality Assurance

  - Installer qualifications: Engage an installer who has no less than 3 years’ experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer’s qualifications: Not less than 5 years’ experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA, OSHA, Life Safety, OSHPD, ADA and CA 01350.

  - Fire performance characteristics: Provide wood and metal components tested in accordance with ASTM E84 for Class A/1 fire characteristics.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture, and physical properties.

- Delivery, Storage and Handling

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Store materials in original, undamaged packaging in a cool, dry place out of direct sunlight and exposure to the elements. A minimum room temperature of 40°F (4°C) and a maximum of 100°F (38°C) should be maintained.

  - Material must be stored flat.

- Project Conditions

  - Materials must be acclimated in an environment of 65°-75°F (18°-24°C) for at least 24 hours prior to beginning the installation.

  - Installation areas must be enclosed and weatherproofed before installation commences.

- Warranty

  - Acrovyn 5-year Limited Warranty

- Applies to Interior Wall Protection orders that do not include recommended components or accessories

  - Assemblies = Brackets, Hardware

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

  - Limited Lifetime Systems Warranty

- Applies to CS Interior Wall Protection projects that include all recommended components and accessories related to CS Interior Wall Protection Products.

  - Assemblies = Brackets, Hardware

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

Part 2 - Products

- Manufacturers

  - Interior surface protection products specified herein and included on the submittal drawings shall be manufactured by Construction Specialties, Inc., 3 Werner Way, Lebanon, NJ 08833 USA 800-233-8493; email: cet@c-sgroup.com

  - Drawings and specifications are based on manufacturer’s literature from Construction Specialties, Inc. drawings and specifications unless otherwise indicated. Other manufacturers must be approved equal by Architect/Owner.

- Materials

  - Solid Wood Components: Shall be manufactured from plain sawn, FAS grade hardwood, kiln dried to a moisture content of 6% to 10%.

  - Stainless Steel: Cast brackets to be type 304 alloy with #4 satin finish. Minimum strength and durability properties as specified in ASTM A240.

  - Fasteners: All fasteners to be non-corrosive. All necessary fasteners to be supplied by the manufacturer.

- Handrails

  - Handrails to be CS Acrovyn: Surface mounted handrail configuration with mounting brackets spaced as indicated on installation instructions. Attachment hardware shall be appropriate for wall construction.

    - Model P-OW  5 1/2" (139.4mm) high configuration consisting of oval wood handrail with matching end caps returning to the wall. Select handrail from Renaissance™ Wood species and finishes. Custom woods and finishes available. Single cantilevered mounting brackets to be stainless steel available with optional powder coat; select from standard powder coat finishes.

    - Model P-RW  5 7/8" (148.9mm) high configuration consisting of round wood handrail with matching end caps returning to the wall. Select handrail from one of Renaissance™ Wood species and finishes. Custom woods and finishes available. Single cantilevered mounting brackets to be stainless steel. For optional stainless steel end caps specify P-RWSS. Optional 90-degree stainless steel inside and outside corners and 3" (76.2mm) splices available. All stainless-steel components available with optional powder coat; select from standard powder coat finishes.

- Finishes

  - All wood components shall be factory finished. Wood components to be final coated with water based, high solids, clear lacquer using a two-coat process. Finish shall be in accordance with specified AWI finish system. Coverage shall be a minimum of 3-5 mils. Gloss shall be measured on 60° gloss meter as per ASTM D523.

  - General: Comply with NAAMM “Metal Finishes Manual” for recommendations relative to applications and designations of finishes.

- Fabrication

  - General: Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish, and member sizes.

  - Preassemble wood components in shop as much as possible to minimize field assembly.

  - Fabricate components with wood joints lightly chamfered. Provide surfaces free of chipping, dents, and other imperfections.

Part 3 - Execution

- Examination

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- Preparation

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris, and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

- Installation

  - Install the work of this section in strict accordance with the manufacturer's recommendations using only approved mounting hardware and locating all components firmly into position, level and plumb.

  - Temperature at the time of installation must be between 65°-75°F (18°-24°C) and be maintained for at least 48 hours after the installation.

- Cleaning

  - General: Immediately upon completion of installation, clean rails, and accessories in accordance with manufacturer’s recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- Protection

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.