CS Acrovyn® Models LG-300, LG-250, LG-200, LG-150, LG-118, and LG-034

Suggested Specifications | Section 10 26 00

### Part 1 - General

- Summary

  - This section includes the following types of wall protection systems:

    - Corner Guards

  - Related sections: The following sections contain requirements related to this section:

    - Handrails, Bumper Guards, Crash Rails, Accent Rails, Wall Covering, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

    - Blocking in walls for fasteners; refer to section 09 22 00 “Supports for Plaster and Gypsum Board”

- References

  - National codes (IBC, UBC, SBCCI, BOCA and Life Safety)

  - American Society for Testing and Materials (ASTM)

- Submittals

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of corner guards. Show methods of attachment to adjoining construction.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of guard:

6" (152.4mm) long sample of each model specified.

  - Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- Quality Assurance

  - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer’s qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA and Life Safety.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- Delivery, Storage and Handling

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Store materials in original, undamaged packaging in a cool, dry place out of direct sunlight and exposure to the elements. A minimum room temperature of 40°F (4°C) and a maximum of 100°F (38°C) should be maintained.

  - Material must be stored flat.

- Project Conditions

  - Materials must be acclimated in an environment of 65°-75°F (18°-24°C) for at least 24 hours prior to beginning the installation.

  - Installation areas must be enclosed and weatherproofed before installation commences.

- Warranty

  - Acrovyn 5-year Limited Warranty

- Applies to Interior Wall Protection orders that do not include recommended components or accessories

  - Assemblies = Brackets, Hardware

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

  - Limited Lifetime Systems Warranty

- Applies to CS Interior Wall Protection projects that include all recommended components and accessories related to CS Interior Wall Protection Products.

  - Assemblies = Brackets, Hardware

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

### Part 2 – Products

2.01 Manufacturer

A. Construction Specialties, 3 Werner Way, Lebanon, NJ 08833 USA 800-233-8493;

email: cet@c-sgroup.com

B. Drawings and specifications are based on manufacturer's literature from

Construction Specialties, Inc. unless otherwise indicated. Other manufacturers must be approved equal by Architect/Owner.

- Materials

  - Clear Polycarbonate: Corner guards shall be extruded from polycarbonate material with an impact resistance of 16 ft. lb./inch as tested per ASTM D-256, Notched Izod Test. Nominal thickness to be .050" (1.3mm) for 3/4" (19.1mm) legs; .075" (1.9mm) for 1-1/8" (28.6mm) and 1 1/2" (38.1mm) legs; and .10" (2.5mm) for 2" (50.8mm), 2 1/2" (63.5mm) and 3" (76.2mm) legs.

  - Fasteners: All fasteners to be non-corrosive and compatible with polycarbonate. All necessary fasteners to be supplied by the manufacturer.

- Corner Guards

  - Clear polycarbonate corner guards by CS Acrovyn: Surface mounted polycarbonate corner guards mounted with screws. Attachment hardware shall be appropriate for wall construction.

    - Model LG-300  90° surface mounted polycarbonate corner guard with 3" (76.2 mm) legs. Specify model LG-250 for 2 1/2" (63.5mm) legs, LG-200 for 2" (50.8mm) legs, LG-150 for 1 1/2" (38.1mm) legs, LG-118 for 1 1/8" (28.6mm) and LG-034 for 3/4" (19.1mm) legs. Odd angles available; specify LG-300M, LG-250M, LG-200M, LG-150M or LG-118M.

- Fabrication

  - General: Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish and member sizes.

# Part 3 - Execution

- Examination

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

  - Do not proceed until unsatisfactory conditions have been corrected.

- Preparation

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer’s instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

- Installation

  - Install the work of this section in strict accordance with the manufacturer’s recommendations, using only approved mounting hardware, and locating all components firmly into position, level and plumb.

  - Temperature at the time of installation must be between 65°-75°F (18°-24°C) and be maintained for at least 48 hours after the installation.

- Cleaning

  - General: Immediately upon completion of installation, clean guards in accordance with manufacturer’s recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- Protection

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.