# CS Acrovyn by Design® Wall Panels Demountable Sure Snap™

Unfinished Edge with Trim

Suggested Specifications | Section 10 26 23

Part 1 - General

- Summary

  - This section includes the following types of wall protection systems:

    - Wall Panels

  - Related sections: The following sections contain requirements related to this section:

    - Handrails, Corner Guards, Bumper Guards, Crash Rails, Accent Rails, Wall Covering, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

- References

  - National codes (IBC, UBC, SBCCI, BOCA and Life Safety)

  - American Society for Testing and Materials (ASTM)

  - Underwriters Laboratories (UL)

- Submittals

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of wall panel products.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, texture, pattern and thickness:

    - Sample of each product specified.

  - Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- Quality Assurance

  - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of wall panels similar in complexity to those required for this project.

  - Manufacturer's qualifications: Not less than 5 years experience in the production of specified products and a record of successful performance.

  - Code compliance: Assemblies should conform to all applicable codes including IBC, UBC, SBCCI, BOCA and Life Safety.

  - Impact Strength: Provide assembled wall protection units that have been tested in accordance with the applicable provisions of ASTM F476.

  - Chemical and stain resistance: Provide wall panels with chemical and stain resistance in accordance with ASTM D543.

  - Color match: Provide wall protection components that are computer controlled within manufacturing tolerances and typical limitations of digital printing color matching.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- Delivery, Storage and Handling

  - Deliver materials to the project site in unopened packaging clearly labeled to show manufacturer.

  - Store materials in original, undamaged packaging in a clean, dry place out of direct sunlight and exposure to the elements. A room temperature of 50-100ºF (10-38ºC) should be maintained.

  - Materials must be stored flat.

- Project Conditions

  - Materials must be acclimated in an environment of 65-75ºF (18-24ºC) for at least 24 hours prior to beginning the installation.

  - Installation areas must be enclosed and weatherproofed before installation commences.

Part 2 - Products

- Manufacturers

  - Interior surface protection products specified herein and included on the submittal drawings shall be manufactured by Construction Specialties, Inc.

- Materials

  - Engineered PETG: Material should be high-impact Acrovyn 4000 with Suede texture, nominal .040" (1.02mm) thickness. Chemical and stain resistance should be per ASTM D543 standards as established by the manufacturer.

  - Particle Board: 3/8" (9.53mm) thick particle board core.

  - Aluminum: Aluminum trims and optional reveals to be alloy 6063 T5 with clear anodized finish; minimum strength and durability properties as specified in ASTM B221.

- Wall Panels

  - Engineered PETG Wall Panels to be CS Acrovyn by Design: Wall panels to be manufactured of .040" (1.02mm) thick Acrovyn by Design sheet in standard Suede texture, factory-bonded to the face side of a 3/8" (9.53mm) thick particle board core. The backside of the panel to be laminated with a moisture resistant barrier.

    - Acrovyn by Design Wall Panel with Unfinished Edge to include aluminum picture frame [1/4" (6.35mm) visible] or aluminum thin trim [1/16" (1.59mm) visible] factory installed on panel. Standard maximum panel size to be 47" x 119" (1.2m x 3.0m). Select from one of Acrovyn by Design standard pattern colorways or supply custom artwork for embedded digital imagery. Overall panel thicknesses of 3/4" (19.05mm) available with demountable Sure Snap™ System mounting hardware mechanically fastened to wall and preinstalled on panel.

- Reveals

  - Reveal spacing to be as narrow or wide as the following allow:

    - Painted wall.

    - Acrovyn reveals to consist of 1 1/2" (38.1mm) wide strips of .040" (1.02mm) thick sheet.

    - Aluminum reveals to be 1 1/2" (38.1mm) wide x .024" (0.61mm) thick.

- Fabrication

  - General: Fabricate wall panels to comply with requirements indicated for design, dimensions, detail, finish and sizes.

- Finishes

  - General: Comply with NAAMM “Metal Finishes Manual” for recommendations relative to applications and designations of finishes.

- Accessories

  - Acrovyn Wall Panels shall be furnished as a complete packaged system, including appropriate demountable hardware.

Part 3 - Execution

- Examination

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- Preparation

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer's installation instructions.

- Installation

  - Install the work of this section in strict accordance with the manufacturer's recommendations using approved demountable hardware.

  - Temperature at the time of installation must be between 65-75ºF (18-24ºC) and be maintained for at least 48 hours after the installation to allow for proper adhesive set up.

  - Relative humidity shall not exceed 80%.

  - Do not expose wall panels to direct sunlight during or after installation. This will cause the surface temperature to rise, which in turn will cause bubbles and delamination.

- Cleaning

  - General: Immediately upon completion of installation, clean wall panels and accessories in accordance with manufacturer's recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- Protection

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.