# | Suggested Specifications | Section 10 26 00

# CS Acrovyn® Model HB-60B

Part 1 - General

- Summary

  - This section includes the following types of wall protection systems:

    - Bumper Guards

  - Related sections: The following sections contain requirements related to this section:

    - Handrails, Corner Guards, Crash Rails, Accent Rails, Wall Covering, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

    - Blocking in walls for fasteners; refer to section 09 22 00 “Supports for Plaster and Gypsum Board”

- Submittals

General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”:

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Shop drawings showing locations, extent and installation details of bumper guards. Show methods of attachment to adjoining construction.

  - Samples for verification purposes: Submit the following samples, as proposed for this work, for verification:

    - 12" (304.8mm) long sample of each model specified.

- Quality Assurance

  - Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer’s qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- Delivery, Storage and Handling

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Material must be stored flat.

- Project Conditions

  - Installation areas must be enclosed and weatherproofed before installation commences.

- Warranty

  - Acrovyn 5-year Limited Warranty

- Applies to Interior Wall Protection orders that do not include recommended components or accessories

  - Assemblies = Brackets, Hardware

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

  - Limited Lifetime Systems Warranty

- Applies to CS Interior Wall Protection projects that include all recommended components and accessories related to CS Interior Wall Protection Products.

  - Assemblies = Brackets, Hardware

  - Accessories = Primer, Adhesive, Caulk, Trims & Moldings

Part 2 - Products

- Manufacturers

  - Interior surface protection products specified herein and included on the submittal drawings shall be manufactured by Construction Specialties, Inc., 3 Werner Way, Lebanon, NJ 08833 USA 800-233-8493; email: cet@c-sgroup.com

  - Drawings and specifications are based on manufacturer’s literature from Construction Specialties, Inc. drawings and specifications unless otherwise indicated. Other manufacturers must be approved equal by Architect/Owner.

- Materials

  - Rubber: To be ethylene propylene diene monomer (EPDM) rubber.

  - Aluminum: To be 6063-T6 aluminum strip.

  - All necessary fasteners to be supplied by the manufacturer.

- Bumper Guards

  - Heavy duty rubber bumper guards to be CS Acrovyn: Surface mounted guards to be EPDM rubber.

    - Model HB-60B  Rubber bumper guard 2 3/8" (60.0mm) high x 1 3/16" (30.0mm) thick supplied in Black with aluminum accent strip. Mounted with stainless steel screws.

- Fabrication

  - General: Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish and sizes. All based upon required field verified dimensions.

Part 3 - Execution

- Examination

  - Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

    - Do not proceed until unsatisfactory conditions have been corrected.

- Preparation

  - Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

  - Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

- Installation

  - Install the work of this section in strict accordance with the manufacturer's recommendations and the required field verified dimensions.

  - Use only approved mounting hardware and locating all components firmly into position, level and plumb.

- Cleaning

  - General: Immediately upon completion of installation, clean material in accordance with manufacturer’s recommended cleaning method.

  - Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

- Protection

  - Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.