SUGGESTED CANADIAN SPECIFICATIONS

SECTION 09 72 00

C/S Acrovyn® .040" Rigid Sheet

# Part 1 - General 1.01  Summary

- This section includes the following types of wall protection systems:

  - Wall Covering

- Related sections: The following sections contain requirements related to this section:

  - Handrails, Bumper Guards, Crash Rails, Corner Guards, Accent Rails, Wall Panels, Door Protection; refer to section 10 26 00 “Wall and Door Protection”

# 1.02  References

- National Building Code of Canada “NBC” (2010)

- National codes (IBC, UBC, SBCCI, BOCA and Life Safety)

- American Society for Testing and Materials (ASTM)

- Underwriters Laboratories (UL)

# 1.03  Submittals

- General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00 “Submittal Procedures”.

- Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

- Shop drawings showing locations, extent and installation details of wall covering products.

- Samples for verification purposes: Submit the following samples, as proposed for this work, for verification of color, texture, pattern and thickness:

1.  Sample of each product specified.

- Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

- Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

# 1.04  Quality Assurance

- Installer qualifications: Engage an installer who has no less than 3 years experience in installation of systems similar in complexity to those required for this project.

- Manufacturer’s qualifications: Not less than 5 years experience in the production of specified products and a record of successful in-service performance.

- Code compliance: Assemblies should conform to all applicable codes including NBC (2010), IBC, UBC, SBCCI, BOCA and Life Safety.

A.  Fire performance characteristics: Provide Vinyl / Acrylic wall protection system with components tested to Canadian Required CAN/ULC S102.2, “Test for Surface Burning Characteristics of Flooring, Floor Coverings, and Miscellaneous Materials and Assemblies” with results listed below:

- Flame spread: 10

- Smoke developed: 45

- Impact Strength: Provide assembled wall protection units that have been tested in accordance with the applicable provisions of ASTM F476.

- Chemical and stain resistance: Provide wall protection system components with chemical and stain resistance in accordance with ASTM D-1308.

- Color match: Provide wall protection components that are color matched in accordance with the following:

1.  Delta Ecmc of no greater than 1.0 using CIELab color space. (Specifier note: Construction Specialties’ colors are matched under cool white fluorescent lighting and computer controlled within manufacturing tolerances. Color may vary if alternate lighting sources are present).

- Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

# 1.05  Delivery, Storage and Handling

- Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

- Store materials in original, undamaged packaging in a clean, dry place out of direct sunlight and exposure to the elements. A minimum room temperature of 40ºF (4ºC) and a maximum of 100ºF (38ºC) should be maintained. C.  Materials must be stored flat.

# 1.06  Project Conditions

- Materials must be acclimated in an environment of 65-75ºF (18-24ºC) for at least 24 hours prior to beginning the installation.

- Installation areas must be enclosed and weatherproofed before installation commences.

# Part 2 - Products 2.01  Manufacturers

A.  Interior surface protection products specified herein and installed on the submittal drawings shall be manufactured by Construction Specialties, Inc.

# 2.02  Materials

A.  Vinyl/Acrylic: Rigid sheet should be high impact Acrovyn, nominal .040" (1.02mm) thickness in Suede texture. Supplied in 4' x 8' or 10' (1.22m x 2.44m or 3.05m) sheet sizes. Available in (60) Acrovyn solid colors or (16) Chameleon patterned colorways which include (14) woodgrains and (2) metals. Specify color-matched caulk or vinyl/acrylic trim as needed for joint/transitions. Metal trims also available.

# 2.03  Fabrication

A.  General: Fabricate wall covering to comply with requirements indicated for design, dimensions, detail, finish and sizes.

# 2.04  Accessories

A.  Acrovyn Wall Covering shall be furnished as a complete packaged system, containing all primers and adhesive. Primer and adhesive materials shall be water based and nonhazardous.

# Part 3 - Execution 3.01  Examination

A.  Verification of conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper or timely completion.

1.  Do not proceed until unsatisfactory conditions have been corrected.

# 3.02  Preparation

- Surface preparation: Prior to installation, clean substrate to remove dirt, debris and loose particles. Perform additional preparation procedures as required by manufacturer's instructions.

- Protection: Take all necessary steps to prevent damage to material during installation as required in manufacturer's installation instructions.

# 3.03  Installation

- Install the work of this section in strict accordance with the manufacturer's recommendations, using approved adhesive.

- Temperature at the time of installation must be between 65-75ºF (18-24ºC) and be maintained for at least 48 hours after the installation to allow for proper adhesive set up.

- Relative humidity shall not exceed 80%.

- Do not expose wall covering to direct sunlight during or after installation. This will cause the surface temperature to rise, which in turn will cause bubbles and delamination.

# 3.04  Cleaning

- General: Immediately upon completion of installation, clean wall covering and accessories in accordance with manufacturer's recommended cleaning method.

- Remove surplus materials, rubbish and debris resulting from installation as work progresses and upon completion of work.

# 3.05  Protection

A.  Protect installed materials to prevent damage by other trades. Use materials that may be easily removed without leaving residue or permanent stains.